SOP #4 – Post-Milking Cleaning (Robot - Boumatic)

(Question FS31, July 2023 Reference Manual pages 12-5 & 12-6. All shaded areas are required elements of your SOP. You may use your own words to communicate the same intent.)

Purpose:  Ensure that milk is cooling properly and that the equipment is cleaned adequately.

- AMS is programmed to automatically wash with detergent _____ times per day on an ____-hour interval.

  - System is programmed that ____ of the washes are completed with acid and with alkaline.

- Change the milk filter _____ times per day just after wash cycle.

  - Change Filter #1, once a day when milk flow goes through Filter #2.

  - Change Filter #2 once a day when milk flow goes through Filter #1.

- Record the following inspections on the Milking Equipment Sanitation Record Sheet (Record 13):

  - Monthly task, first _________of every month before ______milking, inspect interior of milk claws, pipeline, receive jar, inflations of each AMS, buffer tank, bulk tank etc for signs of build up or poor washing.

  - Check and record the temperature of the hot water or wash water.

- SOP’s for the wash procedures for the bulk milk hauler must be posted on the milk house wall.  Include:

  - how to switch diversion from main tank to buffer tank

  - how to start the wash for the main tank

  - how to start the cooler for the buffer tank

- AMS Unit/Room

  - Hose down stall _____ times per day. Usually done during the wash cycle.

  - Use touch screen to extend the hoses so that exterior of hoses can be scrubbed.

  - Camera lens must be wiped down and then rinsed with water to remove water stone deposits.

  - Clean out vent holes at the claw. Check if the steamer is working.

  - Check if feed is being dispensed.

  - Spray out grippers thoroughly to remove debris.

  - Empty treated/fresh cow milking buckets and wash.

  - Check chemical barrels daily.  If 1/3 left change out for new barrel.

  - Check level of teat dip in teat spray system.

  - Ensure chemical dispensers are working.

- Bulk Tank

  - When possible, inspect the interior of the tank to ensure it was washed and drained properly. Check that there are no deposits on the inner surface. Do this regularly.

  - Bulk milk hauler starts wash system.

    - Sanitizing cycle is the end cycle of the wash cycle.

  - Check CIP system

    - Check chemical barrels daily.  If 1/3 left change out for new barrel.

    - Ensure chemical dispensers are working.

  - Cooler turns on automatically after tank is washed and once milk has reached the bottom of the agitator.  No controls need to be touched.

    - Ensure cooler is working.

- TTR/AMS has alarms: check list to ensure that there were no milk monitoring device alarms (cleaning or milk temperature) in the main and buffer tank or attention lists.

OR

- AMS does not have alarms: check the milk temperature a minimum of twice per day (____ AM & ____ PM), with at least 8 hours between each check and review the history of the milk temperature since the last check and record the milk temperature on Record 12.

- If system alarms identified and/or temperature issues, refer to appropriate Corrective Action Plan on Record 16 and then record date, issue and corrective action taken on Record 17.